DEPTHWIZARD

Complete Mathematical & Physical Formulation Handbook
Comprehensive Theoretical Foundations, Proofs & Geodetic Algorithms

| Author: | Meher Soni |
| --- | --- |
| Institution: | Indian Institute of Technology Madras (IIT Madras) |
| Subject: | Remote Sensing Photogrammetry & Geodetic Computer Vision |
| Classification: | Exhaustive Mathematical Handbook across All 9 Pipeline Modules |

1. Monocular Depth & Tiled Spatial Blending

Implementation Reference: depth/tiled_inference.py

For high-resolution satellite orthophotos exceeding single-pass GPU memory, imagery is partitioned into overlapping tiles of size T × T with stride S = ⌊T · (1 - overlap)⌋.

1.1 2D Hann Window Weighting Matrix

To eliminate seam boundaries across tile overlaps, a 2D Hann weighting window W(y, x) is evaluated as the outer tensor product of two 1D windows:

[Equation 1.1: 2D Hann Window Tensor Formulation] w_y(i) = sin²( π (2i + 1) / (2H) ), i ∈ [0, H-1] w_x(j) = sin²( π (2j + 1) / (2W) ), j ∈ [0, W-1] W(i, j) = w_y(i) ⊗ w_x(j) = w_y(i) · w_x(j)

1.2 Multi-Tile Weighted Blending

For M overlapping tile predictions D_k(x, y) positioned at coordinate offsets (x_{0,k}, y_{0,k}):

[Equation 1.2: Normalized Spatial Disparity Accumulation] D_accum(x, y) = Σ [ W_k(x, y) · D_k(x, y) ] W_accum(x, y) = Σ [ W_k(x, y) ] D_final(x, y) = D_accum(x, y) / ( W_accum(x, y) + ε )

2. Relative Digital Surface Model Normalization (rDSM)

Implementation Reference: calibration/rdsm.py

Relative disparity values D(x, y) are normalized to an invariant scale [0.0, 1.0] using robust percentile bounds to reject sensor anomalies and high-frequency edge outliers:

[Equation 2.1: Robust Percentile Normalization] rDSM(x, y) = clip( (D(x, y) - D_p_low) / (D_p_high - D_p_low + ε), 0.0, 1.0 ) where: • D_p_low = Percentile(D, 0.5%) • D_p_high = Percentile(D, 99.5%) • ε = 10⁻⁷ (prevents division by zero in perfectly flat terrain)

3. Metric Elevation Calibration Engine (Absolute Height)

Implementation Reference: calibration/metric.py, calibration/gcp_calibration.py

3.1 Linear Ordinary Least Squares (OLS)

Given K ≥ 2 Ground Control Points P = {(x_i, y_i, H_i)}_{i=1}^K with sampled predicted disparity D_i = D(x_i, y_i):
H(x, y) = a · D(x, y) + b

[Equation 3.1: Closed-Form OLS Metric Calibration] Cost Function: min_{a, b} L(a, b) = Σ_{i=1}^K ( a · D_i + b - H_i )² Closed-Form Solutions: a = Cov(D, H) / Var(D) = Σ [ (D_i - D_bar)(H_i - H_bar) ] / Σ [ (D_i - D_bar)² ] b = H_bar - a · D_bar where D_bar = (1/K) Σ D_i and H_bar = (1/K) Σ H_i.

3.2 Single-Point Anchor Calibration (K = 1)

When only a single GCP (D_1, H_1) is available, vertical scale is fixed to a regional prior a_prior:

[Equation 3.2: Single-Point Datum Anchor] a = a_prior b = H_1 - a_prior · D_1

3.3 Prior-Regularized Least Squares (λ-Ridge)

When sparse GCPs (K < 4) have low variance, an L2 scale penalty regularizes a toward the physical prior a_prior:

[Equation 3.3: λ-Ridge Analytical Normal Solver] min_{a, b} [ Σ_{i=1}^K ( a · D_i + b - H_i )² + λ ( a - a_prior )² ] Normal Matrix System: [ Σ D_i² + λ Σ D_i ] [ a ] [ Σ D_i H_i + λ a_prior ] [ Σ D_i K ] [ b ] = [ Σ H_i ] Determinant: Δ = K ( Σ D_i² + λ ) - ( Σ D_i )² a = [ K ( Σ D_i H_i + λ a_prior ) - ( Σ D_i )( Σ H_i ) ] / Δ b = [ ( Σ D_i² + λ )( Σ H_i ) - ( Σ D_i )( Σ D_i H_i + λ a_prior ) ] / Δ

3.4 Non-Linear Polynomial Calibration

For non-linear sensor responses across mountainous terrains (K ≥ d + 1 GCPs):
H(x, y) = Σ_{k=0}^d c_k · D(x, y)^k = c_d D^d + c_{d-1} D^{d-1} + ... + c_1 D + c_0

[Equation 3.4: Vandermonde Polynomial Solver] c = ( V^T V )⁻¹ V^T H where V_{i, j} = D_i^{d - j} is the Vandermonde design matrix.

3.5 Monotonic Isotonic Regression Calibration

To enforce strict physical elevation monotonicity (∂H / ∂D ≥ 0) without imposing polynomial curvature:

[Equation 3.5: Monotonic Isotonic Interpolation] Optimization: min_{H_hat} Σ_{i=1}^K ( H_i - H_hat_i )² s.t. H_hat_i ≤ H_hat_j whenever D_i ≤ D_j Piecewise Linear Evaluation: H(x, y) = H_hat_k + [ (H_hat_{k+1} - H_hat_k) / (D_{k+1} - D_k) ] · ( D(x, y) - D_k ) for D_k ≤ D(x, y) ≤ D_{k+1}.

3.6 GCP Residuals & Goodness-of-Fit

[Equation 3.6: Calibration Residual Metrics] residual_i = ( a · D_i + b ) - H_i MAE_GCP = (1/K) Σ_{i=1}^K ¦ residual_i ¦ RMSE_GCP = √[ (1/K) Σ_{i=1}^K ( residual_i )² ]

4. Coarse DEM & SRTM Terrain Datum Anchoring

Implementation Reference: calibration/srtm_anchor.py

4.1 Terrarium RGB Elevation Decoding

AWS / Mapzen terrain tiles encode elevation in 24-bit RGB rasters:

[Equation 4.1: Terrarium RGB Decoding] Elevation (m) = ( R · 256.0 + G + B / 256.0 ) - 32768.0

4.2 Web Mercator & Slippy Map Projection

[Equation 4.2: Slippy Map & Web Mercator Transformation] x_tile = ⌊ (lon + 180°) / 360° · 2^z ⌋ y_tile = ⌊ [ 1 - ln( tan(lat_rad) + sec(lat_rad) ) / π ] · ( 2^z / 2 ) ⌋ EPSG:3857 Metric Coordinates: x_m = lon · ( 20037508.34 / 180° ) y_m = [ ln( tan( (90° + lat) π / 360° ) ) / (π / 180°) ] · ( 20037508.34 / 180° )

4.3 Ground Candidate Extraction & Robust Datum Offset

[Equation 4.3: Robust 25th Percentile Datum Shift] Terrain Mask: M_terrain = { (x, y) ¦ Percentile(D, 1.0%) ≤ D(x, y) ≤ Percentile(D, 25.0%) } Datum Offset Anchoring: b = median( H_DEM(x, y) ¦ (x, y) ∈ M_terrain ) - a · median( D(x, y) ¦ (x, y) ∈ M_terrain ) H(x, y) = a · D(x, y) + b

5. Shadow Geometry & Vertical Structure Relief Extraction

Implementation Reference: shadow_detection.py, shadow/m4_physical_raycast_experiment.py

5.1 Spectral Luminance & Rayleigh Skylight Ratios

[Equation 5.1: Spectral Shadow Luminance & Rayleigh Ratios] Luminance: I = 0.299 R + 0.587 G + 0.114 B BlueRatio: BlueRatio = B / ( (R + G) / 2.0 + 10⁻⁵ ) DiffBR: DiffBR = ( B - R ) / ( B + R + 10⁻⁴ )

5.2 Continuous Shadow Confidence Formulation

[Equation 5.2: Multi-Criteria Shadow Confidence] DarkScore = clip( ( 1.5 · I_thresh - I ) / ( 1.5 · I_thresh + 10⁻⁵ ), 0.0, 1.0 ) BlueScore = clip( ( BlueRatio - 0.80 ) / 0.50, 0.0, 1.0 ) SpectralScore = clip( ( DiffBR + 0.10 ) / 0.40, 0.0, 1.0 ) S_conf(x, y) = DarkScore × ( 0.60 · BlueScore + 0.40 · SpectralScore )

5.3 Shadow Length & Direction Extraction via PCA

For connected component shadow region Ω_k = {(r_i, c_i)}_{i=1}^N:

[Equation 5.3: PCA Shadow Axis Eigendecomposition] μ = (1/N) Σ p_i, Σ = (1/N) Σ ( p_i - μ )( p_i - μ )^T Eigendecomposition: Σ v = λ v ==> λ_max, v_max L_pixel = 4.0 · √[ max( 1.0, λ_max ) ] φ_shadow = atan2( v_max_r, v_max_c ) · ( 180° / π )

5.4 Building Height Trigonometry & Absolute Rooftop Elevation

[Equation 5.4: Physical Shadow Trigonometry] Physical Shadow Length: L_physical = L_pixel · GSD Building Height: h_building = L_physical · tan( θ_elev ) = ( L_pixel · GSD ) · tan( θ_elev ) Absolute Rooftop: H_rooftop = H_ground_datum + h_building = H_ground_datum + ( L_pixel · GSD ) · tan( θ_elev )

5.5 Multivariate Uncertainty & Error Propagation

For height function H(L_px, s, θ) = L_px · s · tan θ:

[Equation 5.5: Multi-Variable Analytical Error Propagation] Partial Derivatives: ∂H / ∂L_px = s · tan θ ∂H / ∂s = L_px · tan θ ∂H / ∂θ = L_px · s · sec² θ = ( L_px · s ) / cos² θ Total Analytical Standard Deviation σ_H: σ_H = √[ ( ∂H/∂L_px · δL_px )² + ( ∂H/∂s · δs )² + ( ∂H/∂θ · δθ_rad )² ] σ_H = √[ ( s tan θ · δL_px )² + ( L_px tan θ · δs )² + ( (L_px · s / cos² θ) · (π / 180°) · δθ_deg )² ]

6. Guided Filter Edge-Preserving Depth Refinement

Implementation Reference: shadow/guided_filter.py

6.1 Grayscale Guided Filter

[Equation 6.1: He et al. Guided Filter Formulation] Local Linear Model: q_i = a_k I_i + b_k, ∀ i ∈ ω_k Cost Function: E(a_k, b_k) = Σ_{i ∈ ω_k} [ ( a_k I_i + b_k - p_i )² + ε a_k² ] Parameters: a_k = [ (1/¦ω¦) Σ I_i p_i - μ_I μ_p ] / ( σ_I² + ε ) b_k = μ_p - a_k μ_I Filtered Output: q_i = a_bar_i I_i + b_bar_i where a_bar_i = (1/¦ω¦) Σ_{k ∈ ω_i} a_k and b_bar_i = (1/¦ω¦) Σ_{k ∈ ω_i} b_k.

6.2 3-Channel Color Guided Filter

[Equation 6.2: 3-Channel Matrix Covariance Formulation] q_i = a_k^T I_i + b_k = a_{r,k} R_i + a_{g,k} G_i + a_{b,k} B_i + b_k a_k = ( Σ_k + ε I_3 )⁻¹ Cov_{I, p} b_k = μ_{p,k} - a_k^T μ_{I,k} q_i = a_bar_i^T I_i + b_bar_i

6.3 Color-Edge Gated Disparity Sharpening

[Equation 6.3: Color-Edge Gated Disparity Sharpening] CIELAB Color Gradient Magnitude: ¦¦ ∇ I_LAB ¦¦ = √[ (∂L*/∂x)² + (∂L*/∂y)² + (∂a*/∂x)² + (∂a*/∂y)² + (∂b*/∂x)² + (∂b*/∂y)² ] Edge Gating Weight: W_edge = clip( ¦¦ ∇ I_LAB ¦¦ / ( Percentile_{95}( ¦¦ ∇ I_LAB ¦¦ ) + 10⁻⁵ ), 0.0, 1.0 ) Sharpened Disparity: D_sharp = D + α_strength · ( D - BoxFilter(D, 3×3) ) · W_edge

7. Geomorphometry & Topographic Shaded Relief

Implementation Reference: process_image.py

7.1 Surface Gradient & Slope Angle

[Equation 7.1: Surface Gradient & Slope Formulation] g_x = ( ∂H / ∂x ) · ( 1 / GSD_x ), g_y = ( ∂H / ∂y ) · ( 1 / GSD_y ) Slope (deg) = arctan( √( g_x² + g_y² ) ) · ( 180° / π ) ∈ [0°, 90°]

7.2 Horn's Algorithm for Shaded Relief (Hillshade)

[Equation 7.2: Horn's Shaded Relief Formulation] Terrain Aspect: Aspect = atan2( -g_y, -g_x ) Solar Math Azimuth: φ_math = radians( 360° - φ_azimuth_geo + 90° ) Illumination Shading: Shaded = sin( θ_altitude ) cos( Slope ) + cos( θ_altitude ) sin( Slope ) cos( φ_math - Aspect ) Hillshade = ⌊ 255.0 · clip( Shaded, 0.0, 1.0 ) ⌋

7.3 Heuristic Reliability / Confidence Mapping

[Equation 7.3: Composite Surface Confidence Formulation] Boundary Factor: C_boundary(x, y) = clip( min( x, W-1-x, y, H-1-y ) / Margin_border, 0.35, 1.0 ) Shadow Factor: C_shadow(x, y) = 1.0 - 0.35 · S_conf(x, y) Gradient Factor: C_gradient(x, y) = clip( 1.0 - 0.20 · [ ¦¦∇D¦¦ / ( 2.0 · Percentile_{95}(¦¦∇D¦¦) ) ], 0.50, 1.0 ) Composite Quality: C(x, y) = 0.90 · C_boundary(x, y) · C_shadow(x, y) · C_gradient(x, y) ∈ [0.0, 1.0]

8. Statistical Verification & Benchmark Metrics

Implementation Reference: evaluation/metrics.py

For predicted raster P = {P_i}_{i=1}^N and LiDAR reference DSM R = {R_i}_{i=1}^N:

[Equation 8.1: Complete Benchmark Verification Metrics] 1. Mean Absolute Error: MAE = (1/N) Σ_{i=1}^N ¦ P_i - R_i ¦ 2. Root Mean Squared Error: RMSE = √[ (1/N) Σ_{i=1}^N ( P_i - R_i )² ] 3. Pearson Correlation: r = Σ [ (P_i - P_bar)(R_i - R_bar) ] / √[ Σ (P_i - P_bar)² · Σ (R_i - R_bar)² ] 4. Coefficient of Determination: R² = 1.0 - [ Σ ( P_i - R_i )² ] / [ Σ ( R_i - R_bar )² ] 5. Pixel-wise Absolute Error Map: E(x, y) = ¦ P(x, y) - R(x, y) ¦

9. The Zero-Fabrication Safety Contract

To enforce scientific integrity and eliminate hallucinated metric measurements in uncalibrated imagery:

[Equation 9.1: Piecewise Zero-Fabrication Safety Function] Output Model = ⎧ H(x, y) = a · D(x, y) + b ∈ ℝ (metres), if GCP or DEM calibration supplied ⎨ H_roof = H_datum + L · s · tan θ ∈ ℝ (metres), if solar angles & GSD are present ⎩ rDSM(x, y) ∈ [0.0, 1.0] (unitless), if uncalibrated (Zero-Fabrication Mode)